RATU Network Implementation Guide

Phases 4 · 5 · 6

Device Configuration · Security · Automation · AI & Cloud · Documentation

Prepared by: Christian

Rwanda Advanced Technology University (RATU)

June 2026

# Phase 4 — Device Configuration & Core Services

This phase covers the complete Cisco IOS configuration of all network devices across the three RATU campuses. All commands are entered in Cisco Packet Tracer using the CLI. Follow the build order exactly — VLANs first, then SVIs, trunks, access ports, OSPF, DHCP, and finally NAT.

## 4.1 Build Order

Always follow this sequence. Skipping steps causes connectivity failures that are difficult to debug:

- Basic security configuration on every device

- Create VLANs on all switches

- Configure SVIs (inter-VLAN routing) on Layer 3 switches

- Configure trunk ports between all switches

- Configure access ports for end devices

- Configure OSPF routing between campuses

- Configure DHCP pools for all VLANs

- Configure NAT on border routers

- Test and verify all connectivity

## 4.2 Basic Security (All Devices)

Apply this to every switch and router before any other configuration. Replace the hostname to match the device name.

enable configure terminal hostname RATU-CORE-SW1 no ip domain-lookup enable secret R@TU_C0re2026 service password-encryption banner motd #Unauthorized access is prohibited. Disconnect now.# line console 0 password Console@RATU2026 login exit line vty 0 15 password Remote@RATU2026 login transport input ssh exit end

## 4.3 VLAN Creation — Kigali Core Switch

Create all 9 VLANs on the core switch first. Distribution and access switches will learn VLANs through trunk links.

enable configure terminal vlan 10 name ADMINISTRATION vlan 20 name FINANCE vlan 30 name ACADEMIC_STAFF vlan 40 name STUDENTS vlan 50 name RESEARCH vlan 60 name DATA_CENTER vlan 70 name IOT_DEVICES vlan 80 name GUEST_WIFI vlan 99 name NETWORK_MANAGEMENT end show vlan brief

## 4.4 SVI Configuration — Inter-VLAN Routing

Each VLAN needs a Switched Virtual Interface (SVI) on the Layer 3 core switch. The SVI IP address becomes the default gateway for all devices in that VLAN. The ip routing command is mandatory — without it, traffic will not route between VLANs.

configure terminal interface vlan 10 ip address 10.1.10.1 255.255.255.0 no shutdown exit interface vlan 20 ip address 10.1.20.1 255.255.255.0 no shutdown exit interface vlan 30 ip address 10.1.30.1 255.255.255.0 no shutdown exit interface vlan 40 ip address 10.1.40.1 255.255.252.0 no shutdown exit interface vlan 50 ip address 10.1.50.1 255.255.254.0 no shutdown exit interface vlan 60 ip address 10.1.60.1 255.255.255.0 no shutdown exit interface vlan 70 ip address 10.1.70.1 255.255.255.0 no shutdown exit interface vlan 80 ip address 10.1.80.1 255.255.252.0 no shutdown exit interface vlan 99 ip address 10.1.99.1 255.255.255.0 no shutdown exit ip routing end show ip interface brief

## 4.5 Trunk Port Configuration

Trunk ports carry all VLANs between switches. Every link between two switches must be configured as a trunk on both ends.

configure terminal interface GigabitEthernet0/1 switchport mode trunk switchport trunk allowed vlan 10,20,30,40,50,60,70,80,99 description LINK_TO_DIST_SW1 no shutdown exit interface GigabitEthernet0/2 switchport mode trunk switchport trunk allowed vlan 10,20,30,40,50,60,70,80,99 description LINK_TO_DIST_SW2 no shutdown exit end

## 4.6 Access Port Configuration

Access ports connect end devices (PCs, servers, printers) to the network. Each port belongs to exactly one VLAN.

configure terminal ! --- Administration ports (VLAN 10) --- interface range FastEthernet0/1 - 8 switchport mode access switchport access vlan 10 no shutdown exit ! --- Finance ports (VLAN 20) --- interface range FastEthernet0/9 - 12 switchport mode access switchport access vlan 20 no shutdown exit ! --- Academic Staff ports (VLAN 30) --- interface range FastEthernet0/13 - 18 switchport mode access switchport access vlan 30 no shutdown exit ! --- Student ports (VLAN 40) --- interface range FastEthernet0/19 - 24 switchport mode access switchport access vlan 40 no shutdown exit end

## 4.7 OSPF Dynamic Routing

OSPF automatically discovers and shares routes between all campuses. Use a unique router-id on every device. The wildcard mask is the inverse of the subnet mask.

! --- Kigali Core Switch --- configure terminal router ospf 1 router-id 1.1.1.1 network 10.1.0.0 0.0.255.255 area 0 network 10.255.1.0 0.0.0.3 area 0 network 10.255.2.0 0.0.0.3 area 0 exit end ! --- Musanze Border Router (router-id 2.2.2.2) --- ! network 10.2.0.0 0.0.255.255 area 0 ! network 10.255.1.0 0.0.0.3 area 0 ! --- Huye Border Router (router-id 3.3.3.3) --- ! network 10.3.0.0 0.0.255.255 area 0 ! network 10.255.2.0 0.0.0.3 area 0 show ip ospf neighbor show ip route

## 4.8 DHCP Configuration

DHCP is configured on the Kigali core switch. Exclude the first 50 addresses in large VLANs (first 20 in smaller VLANs) to reserve IPs for servers and printers.

configure terminal ip dhcp excluded-address 10.1.10.1 10.1.10.20 ip dhcp pool ADMIN_KGL network 10.1.10.0 255.255.255.0 default-router 10.1.10.1 dns-server 10.1.60.10 domain-name ratu.edu.rw exit ip dhcp excluded-address 10.1.20.1 10.1.20.20 ip dhcp pool FINANCE_KGL network 10.1.20.0 255.255.255.0 default-router 10.1.20.1 dns-server 10.1.60.10 domain-name ratu.edu.rw exit ip dhcp excluded-address 10.1.40.1 10.1.40.50 ip dhcp pool STUDENTS_KGL network 10.1.40.0 255.255.252.0 default-router 10.1.40.1 dns-server 10.1.60.10 domain-name ratu.edu.rw exit ip dhcp excluded-address 10.1.30.1 10.1.30.20 ip dhcp pool ACADEMIC_KGL network 10.1.30.0 255.255.255.0 default-router 10.1.30.1 dns-server 10.1.60.10 domain-name ratu.edu.rw exit end show ip dhcp pool show ip dhcp binding

## 4.9 NAT Configuration — Border Router

NAT (Network Address Translation) allows all internal devices to share one public IP address for internet access. PAT (Port Address Translation) is used, also called NAT overload.

configure terminal access-list 1 permit 10.0.0.0 0.255.255.255 interface GigabitEthernet0/0 description LINK_TO_INTERNAL_NETWORK ip nat inside no shutdown exit interface GigabitEthernet0/1 description LINK_TO_ISP ip nat outside no shutdown exit ip nat inside source list 1 interface GigabitEthernet0/1 overload end show ip nat translations

## 4.10 Phase 4 Verification Commands

| Command | What It Verifies |
| --- | --- |
| show vlan brief | All 9 VLANs exist and ports are assigned correctly |
| show ip interface brief | All SVIs show up/up with correct IP addresses |
| show ip route | OSPF routes from other campuses appear (marked O) |
| show ip ospf neighbor | Neighbor relationships formed with Musanze and Huye |
| show ip dhcp binding | Devices are receiving IP addresses automatically |
| show ip nat translations | NAT is translating internal IPs for internet access |
| ping 10.1.20.1 | Cross-VLAN connectivity works (Admin to Finance gateway) |
| ping 10.2.40.1 | Inter-campus connectivity works (Kigali to Musanze) |

# Phase 5 — Security, Zero Trust & Automation

Phase 5 transforms the working network from Phase 4 into a secure, Zero Trust infrastructure. Every configuration in this phase is applied on top of the existing working network — do not start Phase 5 until all Phase 4 tests pass.

## 5.1 SSH Secure Administration

Telnet transmits passwords in plaintext. SSH encrypts all administrative sessions. Apply this to every device.

configure terminal ip domain-name ratu.edu.rw crypto key generate rsa ! When prompted: enter 2048 username netadmin privilege 15 secret StrongAdmin@RATU2026 line vty 0 15 login local transport input ssh exit ip ssh version 2 end ! Test from another device: ssh -l netadmin 10.1.99.1

## 5.2 Access Control Lists (ACLs)

ACLs enforce the Zero Trust access policy by blocking unauthorized traffic between VLANs. Apply ACLs to the VLAN interface in the inbound direction.

! --- Block students from Finance and Data Center --- configure terminal ip access-list extended STUDENT_RESTRICTIONS deny ip 10.1.40.0 0.0.3.255 10.1.20.0 0.0.0.255 deny ip 10.1.40.0 0.0.3.255 10.1.60.0 0.0.0.255 deny ip 10.1.40.0 0.0.3.255 10.1.99.0 0.0.0.255 permit ip any any exit interface vlan 40 ip access-group STUDENT_RESTRICTIONS in exit ! --- Block Guest WiFi from all internal VLANs --- ip access-list extended GUEST_RESTRICTIONS deny ip 10.1.80.0 0.0.3.255 10.0.0.0 0.255.255.255 permit ip any any exit interface vlan 80 ip access-group GUEST_RESTRICTIONS in exit end show ip access-lists

## 5.3 Port Security

Port security prevents unauthorized devices from connecting to switch ports by limiting the number of MAC addresses allowed per port. Sticky learning automatically records the MAC addresses of connected devices.

configure terminal interface range FastEthernet0/1 - 24 switchport mode access switchport port-security switchport port-security maximum 2 switchport port-security violation restrict switchport port-security mac-address sticky no shutdown exit end show port-security interface FastEthernet0/1

## 5.4 Site-to-Site VPN — Kigali to Musanze

IPSec VPN encrypts all traffic between campuses. Configure on both border routers. The pre-shared key must match on both ends.

! --- On RATU-BORDER-R1 (Kigali) --- configure terminal crypto isakmp policy 10 encr aes hash sha authentication pre-share group 2 exit crypto isakmp key RATU_VPN_Key2026 address 10.255.1.2 crypto ipsec transform-set RATU-SET esp-aes esp-sha-hmac crypto map RATU-VPN 10 ipsec-isakmp set peer 10.255.1.2 set transform-set RATU-SET match address VPN_TRAFFIC exit ip access-list extended VPN_TRAFFIC permit ip 10.1.0.0 0.0.255.255 10.2.0.0 0.0.255.255 exit interface GigabitEthernet0/1 crypto map RATU-VPN exit end

## 5.5 Python Automation Scripts

These scripts run on your Windows PC with Python 3 and the netmiko library installed. They connect to network devices via SSH and automate repetitive management tasks.

### 5.5.1 Automated Configuration Backup

from netmiko import ConnectHandler from datetime import datetime import os devices = [ {'name':'CORE-SW1', 'host':'10.1.99.2', 'device_type':'cisco_ios'}, {'name':'DIST-SW1', 'host':'10.1.99.3', 'device_type':'cisco_ios'}, {'name':'BORDER-R1', 'host':'10.1.99.1', 'device_type':'cisco_ios'}, ] os.makedirs('backups', exist_ok=True) for d in devices: try: conn = ConnectHandler(**d, username='netadmin', password='StrongAdmin@RATU2026', secret='R@TU_C0re2026') conn.enable() cfg = conn.send_command('show running-config') ts = datetime.now().strftime('%Y%m%d_%H%M%S') fname = f"backups/{d['name']}_{ts}.txt" with open(fname,'w') as f: f.write(cfg) print(f'Backed up {d["name"]} -> {fname}') conn.disconnect() except Exception as e: print(f'FAILED {d["name"]}: {e}')

### 5.5.2 Network Health Monitor

from netmiko import ConnectHandler def check_device(host, name): conn = ConnectHandler(device_type='cisco_ios', host=host, username='netadmin', password='StrongAdmin@RATU2026', secret='R@TU_C0re2026') conn.enable() results = {} results['cpu'] = conn.send_command('show processes cpu ¦ include CPU') results['memory'] = conn.send_command('show processes memory ¦ include Processor') results['ospf'] = conn.send_command('show ip ospf neighbor') conn.disconnect() return results devices = [('10.1.99.1','BORDER-R1'), ('10.1.99.2','CORE-SW1')] for host, name in devices: print(f'\n=== {name} ===') data = check_device(host, name) for k,v in data.items(): print(f'[{k}] {v}')

# Phase 6 — AI, Cloud, Testing & Documentation

Phase 6 elevates the RATU project to a professional engineering portfolio standard. It covers AI-based network intelligence design, hybrid cloud architecture, comprehensive testing, and the full documentation package needed for Master's scholarship applications.

## 6.1 AIOps — AI-Based Network Intelligence

AIOps (Artificial Intelligence for IT Operations) uses machine learning to monitor, predict, and automatically respond to network events. For RATU, the AIOps system is designed as a three-layer pipeline:

- Data collection layer — SNMP agents on all devices collect CPU, memory, interface, and traffic statistics every 60 seconds

- Analytics layer — a machine learning model trains on normal baseline data and raises alerts when metrics deviate beyond thresholds

- Response layer — automated scripts react to alerts by rerouting traffic, isolating devices, or notifying the NOC team

Key AIOps functions implemented in the RATU design:

| Function | Description | Trigger Condition |
| --- | --- | --- |
| Anomaly detection | ML model flags unusual traffic patterns | Bandwidth > 2× 7-day average |
| Predictive maintenance | Forecasts device failures before they occur | CPU > 85% for 10+ minutes |
| Automated failover | Switches traffic to backup link automatically | Primary WAN link down |
| Security alerting | Detects port scans and brute-force attempts | 5+ failed SSH logins in 60s |
| Capacity planning | Reports VLANs approaching bandwidth limits | VLAN utilization > 70% |

Table 6.1: AIOps functions, descriptions, and trigger conditions.

## 6.2 Hybrid Cloud Architecture

RATU uses a hybrid approach combining on-premises infrastructure with cloud services. Critical and sensitive data remains on-premises in Kigali; scalable, non-sensitive workloads move to the cloud.

| Resource | Location | Reason |
| --- | --- | --- |
| Student database | On-premises (Kigali) | Data sovereignty and privacy regulations |
| Finance systems | On-premises (Kigali) | Security and compliance requirements |
| Authentication server | On-premises (Kigali) | Zero Trust requires local control |
| Virtual labs | Cloud (AWS/Azure) | Scalable compute for student practical work |
| Backup storage | Cloud (AWS S3) | Offsite disaster recovery |
| Online collaboration | Cloud (Microsoft 365) | Scalability and availability |
| Research HPC workloads | Cloud (burst compute) | On-demand high-performance computing |

Table 6.2: Resource placement decisions for RATU hybrid cloud architecture.

## 6.3 Server Virtualization

Instead of deploying physical servers for each service, RATU uses virtualization to run multiple services on fewer physical hosts. This reduces hardware costs, improves manageability, and enables faster disaster recovery.

| VM Name | Service | Physical Host | vCPUs | RAM | Storage |
| --- | --- | --- | --- | --- | --- |
| VM-DNS | DNS Server | Host-1 | 2 | 4 GB | 50 GB |
| VM-DHCP | DHCP Server | Host-1 | 2 | 4 GB | 50 GB |
| VM-WEB | Web + Admissions | Host-2 | 4 | 8 GB | 200 GB |
| VM-LMS | Moodle LMS | Host-2 | 8 | 16 GB | 500 GB |
| VM-DB | Finance + SIS DB | Host-3 | 8 | 32 GB | 1 TB |
| VM-MAIL | Email Server | Host-3 | 4 | 8 GB | 500 GB |
| VM-MON | Network Monitoring | Host-1 | 4 | 8 GB | 200 GB |
| VM-BACKUP | Backup Services | Host-4 | 2 | 4 GB | 4 TB |

Table 6.3: Virtual machine inventory and resource allocation.

## 6.4 Testing & Validation

All tests must be documented with screenshots and included in the project portfolio. The table below defines the full test plan for the RATU network.

| Test # | Test Name | Command / Method | Expected Result | Pass Criteria |
| --- | --- | --- | --- | --- |
| T-01 | Inter-VLAN routing | ping 10.1.20.1 from VLAN 10 | Reply received | 4/4 replies |
| T-02 | Cross-campus routing | ping 10.2.40.1 from Kigali | Reply received | 4/4 replies |
| T-03 | DHCP assignment | ipconfig on student PC | IP in 10.x.40.x range | IP, mask, GW assigned |
| T-04 | DNS resolution | nslookup lms.ratu.edu.rw | Returns 10.1.60.35 | Correct IP returned |
| T-05 | Student ACL | ping 10.1.20.10 from VLAN 40 | Request timed out | 0/4 replies (blocked) |
| T-06 | Guest isolation | ping 10.1.10.1 from VLAN 80 | Request timed out | 0/4 replies (blocked) |
| T-07 | OSPF adjacency | show ip ospf neighbor | Musanze + Huye listed | FULL state shown |
| T-08 | VPN tunnel | show crypto ipsec sa | Tunnel active | pkts encaps > 0 |
| T-09 | Port security | Connect 3rd device to port | Port shuts down | Violation logged |
| T-10 | SSH access | ssh -l netadmin 10.1.99.1 | Login prompt appears | Session established |
| T-11 | NAT internet | ping 8.8.8.8 from internal | Reply received | 4/4 replies |
| T-12 | Python backup | Run backup.py | Config file saved | File exists with content |

Table 6.4: Full test plan — document results for each test in the portfolio.

## 6.5 GitHub Repository Structure

Upload your project to GitHub using this folder structure. Each folder contains evidence of work completed in that phase.

RATU-Network-Project/ ├── README.md ← Project overview and status ├── docs/ │ ├── RATU_Requirements_v1.0.docx │ ├── RATU_Phase3_IP_Plan.xlsx │ └── RATU_Implementation_Guide.docx ├── Network_Diagrams/ │ ├── physical-topology.png │ ├── logical-topology.png │ ├── vlan-diagram.png │ └── security-zones.png ├── Cisco_Configurations/ │ ├── RATU-CORE-SW1.txt │ ├── RATU-DIST-SW1.txt │ ├── RATU-BORDER-R1.txt │ ├── MUS-CORE-SW1.txt │ └── HUY-CORE-SW1.txt ├── Packet_Tracer_Files/ │ ├── RATU_Practice_Lab.pkt │ └── RATU_Full_Network.pkt ├── Python_Automation/ │ ├── backup.py │ └── monitor.py ├── Security_Policies/ │ └── acl-policies.txt └── Testing_Reports/ ├── test-results.md └── screenshots/

## 6.6 Technical Report Chapter Guide

Your technical report must follow this structure. Each chapter maps directly to a completed phase of the project.

| Chapter | Title | Content Summary | Phase |
| --- | --- | --- | --- |
| 1 | Introduction | Background, problem statement, objectives, scope | Phase 1 |
| 2 | Literature Review | Enterprise networks, Zero Trust, cloud, AIOps — cite 10+ references | Research |
| 3 | System Design | Requirements, architecture, topology, IP plan, security model | Phase 2–3 |
| 4 | Implementation | All CLI configurations, Python scripts, cloud design | Phase 4–5 |
| 5 | Testing & Results | Test table results, screenshots, analysis of findings | Phase 6 |
| 6 | Conclusion | Summary of achievements, challenges faced, future work | Phase 6 |

Table 6.5: Technical report chapter structure mapped to project phases.

## 6.7 Project Completion Checklist

Use this checklist to confirm the project is complete before submitting your scholarship application:

- Phase 1 — Requirements document written and uploaded to GitHub

- Phase 2 — Network topology diagrams created in draw.io

- Phase 3 — IP addressing spreadsheet complete for all 3 campuses

- Phase 4 — Kigali campus fully configured and tested in Packet Tracer

- Phase 4 — Musanze and Huye campuses configured and inter-campus routing tested

- Phase 4 — DHCP and NAT working and verified

- Phase 5 — SSH enabled on all devices, Telnet disabled

- Phase 5 — ACLs blocking students from Finance and Guest from internal VLANs

- Phase 5 — VPN tunnels active between all three campuses

- Phase 5 — Port security configured on all access switches

- Phase 5 — Python backup script tested and working

- Phase 6 — All 12 tests in Table 6.4 passed with screenshots

- Phase 6 — GitHub repository complete with all folders and files

- Phase 6 — Technical report written (6 chapters) and exported as PDF

- Phase 6 — Network diagrams exported cleanly from draw.io